Z-Buffer/FillAreaSets Modification Implementation Notes

January 1993

Release 1.0

X Consortium, Inc.

These notes are provided to help anyone interested in understanding what

modifications were made and how the modified server works.

1. Strategy

HLHSR strategy -- Z-BUFFER

~~~~~~~~~~~~~~

The method I used to implement the code was to make duplicates of each

procedure, which would perform Z-buffering.  The spans that result at level 1

are then sent to a ZBuffer_FillSpans routine which performs the actual

z-buffering, and translates them into normal spans which are sent to the GC

as a normal fillspans operation.  The method used for interpolating

coordinates across the Z-buffer was a Bresenham-like algorithm.

FILL AREA SETS strategy -- SPAN ELIMINATION

~~~~~~~~~~~~~~~~~~~~~~~

The method I used to implement FillAreaSets was to use a procedure that would

obtain the spans that are normally calculated in a FillArea operation for

EACH of the contours that are defined for the FillArea object.  These spans

are then compared against one another.  Whenever a span appears an even number

of times, it is eliminated.  If it appears an odd number of times, it is

still visible.  This required interpolation of locations across the

Z plane as well as the X plane (in order to accomodate Z-buffering).  This

may be visualized as a sort of XOR operation of spans, but it is performed

_prior_ to writing to the screen.

2. Changes to each file

The changes I made to each modified file and the descriptions of each new file

follow:

In level 1 ddpex:

~~~~~~~~~~~~~~~~~

Imakefile	- added new files for supporting ZBuffering and FillAreaSets,

new include directory for files used by Z-buffering level

routines (normally used by ddx level routines).

miLvl1Tab.c	- added a function pointer jump table for HLHSR equivalents of

normal Level1 rendering procedures.

miRndFArea.c	- added HLHSR version of Level1 FillArea rendering procedure.

Also added support for FillAreaSets.

miRndMarkr.c	- added HLHSR version of Level1 Marker rendering procedure.

miRndPLine.c	- added HLHSR version of Level1 PolyLine rendering procedure.

miRndTStrip.c	- added HLHSR version of Level1 TriStrip rendering procedure.

miRndText.c	- added HLHSR version of Level1 Text rendering procedure.

miZ.h		- ddx-level and Z-buffering level external procedure

declarations, also some constants used by line-drawing

procedures.

miZBuffer.c	- Basic Z-buffer management and spanning procedures.

midashZ.c	- ** taken from ddx level and modified to support 3-D and

Z-buffering.

mifillsetsZ.c	- Basic polygon span calculations for FillAreaSets.

mifpolyZ.h	- ** taken from ddx level and modified to support 3-D and

Z-buffering.

mipolyZ.c	- **  ""

mipolyZ.h	- **  ""

mipolyconZ.c	- **  ""

mipolygenZ.c	- **  ""

mipolyutilZ.c	- **  ""

miscanfillZ.h	- **  ""

mispansZ.c	- ** taken from ddx level and modified to support Z-buffering;

added basic span elimination/calculation routines needed for

FillAreaSets

mispansZ.h	- ** taken from ddx level and modified to support 3-D and

Z-buffering.

mistructZ.h	- **  ""

miwidelineZ.c	- **  ""

miwidelineZ.h	- **  ""

mizerlineZ.c	- **  ""

PEX5/ddpex/mi/level2:

~~~~~~~~~~~~~~~~~~~~~

Imakefile	- added new include directory for files used for Z-buffering

support (Z-buffer-active detection).

miFillArea.c	- added support for Z-buffering by calling appropriate Level1

rendering procedure and setting point type appropriately.  Also

added support by FillAreaSets by setting shape parameter

appropriately.

miPolyLine.c	- added support for Z-buffering by calling appropriate Level1

rendering procedure and setting point type appropriately.

miTrans.c	- added support for Z-buffering by allowing for 3DS type output

points.

ddContext.c	- added support for Z-buffering by performing Z-buffer

allocation and by loading the  Z-buffer function pointer jump

table.

miMarkers.c	- added support for Z-buffering by calling appropriate Level1

rendering procedure.

miText.c	- added support for Z-buffering by calling appropriate Level1

rendering procedure and setting point type appropriately.

miTriStrip.c	-   "     "

PEX5/ddpex/mi/level3:

~~~~~~~~~~~~~~~~~~~~~

Imakefile	- added new include directory for files used for Z-buffering

support (Z-buffer-active detection).

miRender.c	- added support for Z-buffering by clearing the Z-buffer when

necessary.

PEX5/ddpex/mi/include:

~~~~~~~~~~~~~~~~~~~~~~

miInfo.h	- changed SI_HLHSR_NUM from "Off" to "ZBuffer".

miRender.h	- added HLHSRProcs lookup table and zbuffer attribute in

the static DDcontext, and ZBUFFER_ACTIVE (a macro that

detects whether Z-buffer is active or not)

PEX5/include:

~~~~~~~~~~~~~

ddpex.h		- fixed a bug in ddCoord3DS structure because it previously

would have caused byte alignment problems.

ddpex3.h	- added support for ddCoord3DS, various 3DS types, and

added the Zbuffer structure definition.

----------------------------

Daiji Takamori

takamori@x.org

January 1993